Welcome to the latest version of the regen design system!

The prior version created a geometry that was too difficult to manufacture, as it varied channel width, channel height and the inner wall non-linearly across the engine. Those scripts are still largely useful and can be found in EngineDesigner_v1. This system will be incorporating much more accurate stress calculations while also varying the parameters in a much more manufacturing-friendly way.

Knobs to turn:

- Channel width

- Fin width

- Channel height

Here’s how it will generally work:

- Create the contour and property array for all 200 points of the nozzle

- Pick four points to test with: Injector face, converging section, throat, nozzle exit

- At each point, simulate with a bunch of fin and channel widths and find the factor of safety at each one.

- Find the lowest pressure drop combination that is still above FOS at the throat. Now channel number is restricted to integer multiples from the throat number

- Find lowest pressure drop combination that is still above FOS. Restrict tests to channel numbers specified by the throat geometry.

- Stitch together the values found at these points into a manufacturable geometry

Detailed architecture:

- Create the contour and property array for all 200 points of the nozzle

- Pick four points to test with: Injector face, converging section, throat, nozzle exit. We’ll call these the “critical stations”

- Define wall thickness = 1 mm and height = 1 mm

- Define range of fin thicknesses and range of channel thicknesses for testing

- Iterate through stations to find total heat flux and thus coolant temperature deltas at each station. Use this to get the bulk temperatures at each critical station

- For each critical station:

  - For each combination of fin and channel widths:

    - Converge on thermal equilibrium:

      - Run Bartz Correlation with initial wall temp guess to get the heat flux

      - Use this to find q_ge and q_ce

      - Move T_wg until they are equal

    - Calculate inner and outer wall temperatures

    - Run stress.py with the given average wall temperature and delta

    - Save FOS’s to an array

  - Plot the FOS against channel and fin widths

Questions/Comments:

- Which specific part of the converging section should be tested? Just picked the midpoint

- SOLVED: If we are finding the lowest gas side temp at each point being tested, the channels will be as thin as can be (assuming the fins stay within thermal constraints) since this smaller hydraulic diameter will make h_c very high. The problem here is that friction is going to start to pile up. What we can do is pick the lowest pressure solution that is still above our FOS.

- What are kd_factor and Kt

- T_wg is currently an educated conservative guess to run Bartz to get gas side heat transfer rates. We could only get it with a convergence in which case each situation would need have a fully defined geometry.

- Is it reasonable to have the fin height change throughout the contour?

  - No

- Can we have a different number of channels between two stations? If so, does each branch need to split/merge into two, or could specific channels branch while others remain as one?

  - No

- From my understanding if the channel width is different between two stations, we’ll have a linear lead-in at some point between them where the width changes. Over how much distance are you expecting one of these transitions to be?

  - Linear connection throughout

- Run analysis at throat, barrel and nozzle to find the lowest pressure drop combination of parameters that is at the desired FOS (Alternatively we could just have it max FOS and only constrain it geometrically).

  - Pick smallest channel and fin widths possible. Increase until FOS is met.

  - Adjust slightly for integer channel number

- Create geometry at each of the 200 stations by linear connections from critical points

- Run full simulation, iterating over all 200 points. Check that FOS is above limit at each point and that coolant is not boiling.

- If coolant is boiling, we need to decrease the convection coefficient and try again. This can be done by increasing channel hydraulic diameter. Increment this until FOS limit is reached.

- If this fails to close then no solution can be found with this approach. This will require a change to engine parameters or the addition of film cooling.

NOTE: There’s essentially no point in increasing fin width... If you do, wall temp will go up as will pressure drop. The only reason you might want to is if you’re approaching boiling, but in that case you can still just futher increase channel width to decrease coolant convection.

Stress calcs

So, the thermal stress would be acting to both compress and expand the wall. Hoop stress would only be acting to compress the wall (since coolant pressure is larger). This means highest stress will be compressive on inside of wall as the sum of these two. Also, look into the relative expansion of the jacket and the steel outer piece, as this would probably cause more pressure on the jacket.

Finally, the thermal stress on the old matlab code was off by a factor of 4.

The relative expansion of the copper and the steel pieces seems to cause the bulk of the stress on the jacket. There is a radial strain of roughly 0.004 on the inner wall since the copper jacket restricted from expanding. From what I can tell from stress-strain graphs for annealed copper this creates a pressure of roughly 60 MPa radially inward. The compressive hoop stress that this would cause is around 3000 MPa which would suggest failure, but what probably happens here is that the copper walls bow out a bit, letting the compressive strain die down.

Main issues:

- Relative expansion of copper and steel pieces would cause some compressive stresses in the copper

- Current thermal stress is based on linear stress strain relationship

Another Overhaul

According to this paper https://www.sciencedirect.com/science/article/pii/S100093611930353X#b0025

The failure on regen designs is in continued reuse which results in a buildup of residual strain. For the engine given, an expected cycle life of 300 is determined which is much higher than we’ll need.

In that case, what we need to worry about then is boiling and very high wall temps that may cause melting or dumb low yield for failure in shear due to pressure (which would be a very low sigma and probably close to melting anyway). I think a good approach here will be to pick a coolant exit temp and then solve for the total heat that can enter the jacket with some factor of safety. From this we can get a reasonable approximation of the coolant temp throughout the contour by assuming it’s linearly increasing. With max total heat transfer found, we can solve for the minimum wall temp difference (setting the temp constant for all stations). Pick a max gas side temp and solve equilibrium for h_c. From this we can get the hydraulic diameter. Since channel height is restricted, we can then solve for channel width.

Do this for each station and check coolant outlet temp. If it’s too high, redo it with a higher gas wall temp, otherwise lower it.

Conclusion:

Solve for throat, barrel and exit dimensions given a gas side wall temp. This comes from solving thermal equilibrium for h_c, from which you can get hydraulic diameter. Construct geometry from these three points and simulate. If coolant exit temp is too high, increase the gas side wall temp and repeat. Otherwise lower the temp to converge on desired exit temp.